# Gateway Web

| Date | Version | Author | Notes |
| --- | --- | --- | --- |
| 10.6.2015 | 1.0 | Eitan Revach |  |
| 13.6.2015 | 1.2 | Eitan Revach |  |
| 14.6.2015 | 1.3 | Eitan Revach | Description states: IDE, CONNECTING, CONNECTED |
| 14-Aug-2015 | 1.4 | Eitan Revach | Minor fixes. |

Table of Contents

API	3

Introduction	3

HTTP Requests	4

1.	General Info	4

2.	All Config	5

3.	Device Name	5

4.	New MAC	6

5.	Set IP Settings	6

6.	Set Serial Settings	6

7.	Set Server Settings	7

8.	Reboot/Apply Settings	7

General Notes:	7

Gateway Functionality	8

Machine states:	8

General Flow/Lifecycle:	9

1.	Just Manufactured.	9

2.	Manufacturing	9

3.	Routine	9

4.	Return to Factory	9

State Description	10

CONNECTED	10

IDLE	10

# API

## Introduction

The UI consists of 2 files:

- Index.html
includes 2 subpages:

  - Preview page – shows general information
uses the call http://gateway_Address/Data/Info to get the data.

  - Setting Page – all settings and factory reset

- favicon.ico

All data in UI returns from simple HTTP GET request to gateway.

## HTTP Requests

- General Info
Returns the data for Preview Page.

  - Request

http://gateway_Address/Data/Info

  - Response

Data structure:

Notes:

    - deviceName – returns internal device Name (can be set). If none – return “Cyber Rain – Gateway”

    - rssiLevel – Optional. Return empty.

    - mac – MAC address. Return Empty if none.

    - serverTotalConnection : total seconds for current connection, if connected.

- All Config
Returns the settings values for Settings Page.

  - Request
 http://gateway_Address/Data/Config

  - Response
Data structure:

Notes:

    - ipconfig

      - addressType – AUTO or STATIC

    - remoteServer
see notes in request “Set Server Settings“

      - serverAddress : can be numbers (212.25.35.68) or DNS (walla.co.il)

      - timeout – seconds.

      - Interval – minutes.

- Device Name
Set new MAC for device.

  - Request
http://gateway_Address/Data/SetName?NewName=X1

  - Response
Respose should contains Boolean value only (true / false), according to results.

- New MAC 
Set new name for device.

  - Request
http://gateway_Address/Data/AllocateMAC?NewMAC=XXXX

    - X1 = MAC address as 00:00:00:00:00:00 (6 HEX values)
Can be NULL – in this case the Device should generate the MAC.

  - Response
Response includes the accepted MAC address. (Device may decline requested MAC and suggest other). If null – means the device rejected the request.

- Set IP Settings  
Set new IP address for device. Notes:

  - Request

http://gateway_Address/Data/SaveIPSettings?type=X1&sip=X2&mip=X3&gip=X4&dip=X4

    - type = AUTO or STATIC

    - sip = static IP (like 10.0.0.3)

    - mip = subnet IP

    - gip = default gateway

    - dip = DNS IP

  - Response
Respose should contains Boolean value only (true / false), according to results.

- Set Serial Settings  
Set new Serial/UART settings for device. Notes:

  - Request

http://gateway_Address/Data/SaveSerialSettings?baudrate=X1&bits=X2&parity=X3&sbits=X4&flow=X4

    - baudrate = values 4800,9600,19200, 38400, 56700, 115200

    - bits = DataBits. Values 7 or 8.

    - parity = Parity. Values: None, Odd, Even.

    - sbits = Stop Bits. Values: 1 or 2.

    - flow = Flow Control. Values: None, Hardware, XonXoff.

  - Response
Respose should contains Boolean value only (true / false), according to results.

- Set Server Settings 
Set the target server to connect.

  - Request
http://gateway_Address/Data/SaveServerSettings?serverAddr=X1&port=X2&interval=X3&timeout=X4&addressType=X5    
Notes:

    - addressType –

      - IP – address contains numbers only (10.0.0.1)

      - DNS – address is DNS type (like walla.co.il)

    - serverAddr – IP or DNS address(according to IP type)

    - port – values 0-65535. Default 50100.

    - interval – minutes. Default 5.
Delay from last disconnection, before new connection attempt.

    - timeout – seconds. Default 300.
Total silence period. When expired reconnect to server.

  - Response
Respose should contains Boolean value only (true / false), according to results.

- Reboot/Apply Settings 
Apply all settings and reboot

  - Request
http://gateway_Address/Data/Apply

  - Response
Respose should contains Boolean value only (true / false), according to results.
Device should reboot.

### General Notes:

- All HTML responses should includes the HTTP headers:
Access-Control-Allow-Origin: *
Access-Control-Allow-Methods: *

- All responses can omit spaces.

- Example application can be found at http://rnd1.glc-service.com/GW

# Gateway Functionality

## Machine states:

| State | Description | LEDS activity |  |
| --- | --- | --- | --- |
| BLANK (Just manufactured) | No settings found for Remote server connection. All settings are default: (AUTO-DHCP, 9600-N-1) | Online – blinks slow TX/RX – blinks slow |  |
| CONNECTING | Trying to connect server | Online – blinks fast |  |
| IDLE (Between connections. Settings available) | All settings were found and valid. Gateway is waiting to next connection (according to Interval settings). | None. |  |
| CONNECTED | Gateway is connected to server by TCP/IP. Timeout still not arrived. | Online – ON TX/RX – according to stream activity. |  |
| STATIC IP MODE | Gateway entered special mode for easy discovery. | Online – OFF. TX/RX – blinks slow separately, one at time. (1sec RX, 1sec TX…) | Temporary state. No connection to server will be made. Exiting this mode after 15 mins or restart. |

## General Flow/Lifecycle:

- Just Manufactured.
Once the code is set in device

  - These are the default settings:

    - AUTO DHCP

    - 9600-N-1

    - No Remote server address/port

    - Interval: 5 (min)

    - Timeout: 300 (seconds)

  - Gateway will not try connecting the server and will not handle UART activity.

### Manufacturing

  - User enters the UI using the STATIC IP MODE:

    - Power ON the gateway while holding the (S2, S22) outer button for 10 seconds.

    - Gateway enters the STATIC IP MODE.

    - Blinks the LED2 (all tree) for 5 seconds)

    - The Gateway will use the

      - IP: 10.0.0.1

      - Default Gateway: 10.0.0.2

    - The gateway will be discoverable in Cross mode (2 PC connected with LAN) as well as in Router (if static IP is allowed).

    - The user will set the Remote Server address and port and other fields, as required.

    - Once the user click on Apply/Reboot – all settings will be treated as Factory Settings.

    - The user will restart the Gateway.

    - After restart – the Gateway will operate in CONNECTING state (with AUTO DHCP and the server settings).

    - Note – erasing these Factory Settings can be done only by re-programming the device.

### Routine

  - After settings are valid the general routine flow is :
IDLE --> CONNECTING  --> CONNECTED --> IDLE

  - After Power ON – device will try connecting immediately.

  - When connected – gateway must ensure the health of connection. In case of silence longer than was set in the Timeout(sec), Gateway will exit CONNECTED state and enters IDLE state.

  - After disconnection – device enters the IDLE state and waits there for X mins, according to Interval settings.

### Return to Factory

  - Holding the (S1) inner button for 20 seconds will set Factory Mode.

  - All settings will be changed to original settings that were set in manufacturing stage.

- Reset

  - Holding the (S1) inner button for 5 seconds will set reset the device.

# State Description

### CONNECTED

Gateway will always try connecting server (using TCP/IP connection).

Once it’s connected, the state changes from CONNECTING to CONNECTED.

While in state CONNECTED – gateway will exit this state in one of the next condition:

- Real disconnection from server.

- No data was received from Server for more than defined in Timeout(seconds).

Data Activity - Every data from UART will be sent to Server and vice versa.

Clearly, if gateway is disconnected – no data handling can be done with UART data.

### IDLE

When gateway it’s not connected and remote server setting are valid, gateway should try connecting the server every Interval(min), as defined. When Interval value is expired the gateway will try connecting the server up to 10 times. If failed, after 10 attempts, it will wait again amount of time as Interval(min).