מסמך הגדרות

בקר GSI

| תאריך | גרסה | הערות |
| --- | --- | --- |
| 8.9.2016 | 1.0 | אלירן חדד |

Contents

רקע טכני/חומרה של הבקר	3

כללי	3

טבלת מיפוי זיכרון לפקודות Write/Read Memory	4

מבני נתונים	5

# רקע טכני/חומרה של הבקר

## כללי

בקר מבוסס מעבד STM32L052.

מוזן במתח של 24AC / 4 סוללות DC, Type C

# טבלת מיפוי זיכרון לפקודות Write/Read Memory

כל הפקודות שעוסקות בבדיקת חומרה מתבצעות על ידי פקודות Write Memory (פקודות 0x78,0x79,0x80 בפרוטוקול).

| כתובות | תיאור | Len | Read / Write |  |
| --- | --- | --- | --- | --- |
| I/O address |  |  |  |  |

# מבני נתונים

פקודות שמגדירות מבני נתונים (כגון כתובת שרת) מתבצעות על ידי פקודת Write CNF (קוד 0x20 בפרוטוקול(

הגדרות מערכת

| CNF / Element Type:0x01 / System |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Comments | Value Ranges | Units | Value type | Name | Len / Type | Offset |
| time limitation for communication | 0-65535 |  | hhh:mm | Modem Cyclic Start Time | Word | 0 |
|  | 0-65535 |  | hhh:mm | Modem Cyclic End Time | Word | 2 |
| time between communications | 0-65535 |  | hhh:mm | Modem Cycle Time | Word | 4 |
| (minutes) time to wait in error, before another try | 0-65535 |  | minutes | Modem Error Cycle Delay | Word | 6 |
| manual times to communicate | 0-65535 |  | hhh:mm | Hourly Comm Wake Up Times[0] | Word | 8 |
| manual times to communicate |  |  |  | Hourly Comm Wake Up Times[1] | Word | 10 |
| manual times to communicate |  |  |  | Hourly Comm Wake Up Times[2] | Word | 12 |
| (seconds) stability On-Delay time | 0-65535 | seconds |  | Cond On Delay[0] | Word | 14 |
| (seconds) stability On-Delay time | 0-65535 | seconds |  | Cond On Delay[1] | Word | 16 |
| (seconds) stability On-Delay time | 0-65535 | seconds |  | Cond On Delay[2] | Word | 18 |
| (seconds) stability On-Delay time | 0-65535 | seconds |  | Cond On Delay[3] | Word | 20 |
| (seconds) stability Off-Delay time | 0-65535 | seconds |  | Cond Off Delay[0] | Word | 22 |
| (seconds) stability Off-Delay time | 0-65535 | seconds |  | Cond Off Delay[1] | Word | 24 |
| (seconds) stability Off-Delay time | 0-65535 | seconds |  | Cond Off Delay[2] | Word | 26 |
| (seconds) stability Off-Delay time | 0-65535 | seconds |  | Cond Off Delay[3] | Word | 28 |
| No of master valve. 0=none 1... [VLV_NUM]= valve |  |  |  | Main Out | Byte | 30 |
| No of master valve 0=none 1-8= valve |  |  |  | Fert Output | Byte | 31 |
| No of master valve. 0=none, 1... [VLV_NUM]= valve |  |  |  | Flash Output | Byte | 32 |
| 0 = no input source 1... [INPUTS] - physical input |  |  |  | Cond Input[0] | Byte | 33 |
| 0 = no input source 1... [INPUTS] - physical input |  |  |  | Cond Input[1] | Byte | 34 |
| 0 = no input source 1... [INPUTS] - physical input |  |  |  | Cond Input[2] | Byte | 35 |
| 0 = no input source 1... [INPUTS] - physical input |  |  |  | Cond Input[3] | Byte | 36 |
| (milliseconds) pulse length for valve command |  |  |  | Out Command Duration | Byte | 37 |
| (seconds) timeout for modem activity |  |  |  | Modem Stop Delay | Byte | 38 |
| maximum tries in modem error |  |  |  | Modem Retry Number | Byte | 39 |
|  |  |  |  | Setup Byte A | Byte | 40 |
| 0 = rainy=> condition off (NC) 1 = rainy=> condition on (NO) |  |  |  | Bit.0 –Rain Switch Direction |  |  |
|  |  |  |  | Bit.1 - |  |  |
|  |  |  |  | Bit.2 |  |  |
|  |  |  |  | Bit.3 - |  |  |
|  |  |  |  | Bit.4 - |  |  |
|  |  |  |  | Bit.5 - |  |  |
|  |  |  |  | Bit.6 |  |  |
| ABS |  |  |  | Battery Low Failure React | Byte | 41 |
|  |  |  |  | Battery Critical Low Failure React | Byte | 42 |
| ABS |  |  |  | Capacitor Charge Failure React | Byte | 43 |
| ABS |  |  |  | Capacitor Discharge Failure React | Byte | 44 |
| ABS |  |  |  | Memory Failure React | Byte | 45 |

הגדרות צינור ראשי / MainPipe

| CNF / Element Type:0x02 / Main pipe |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Comments | Value Ranges | Units | Value type | Name | Len / Type | Offset |
| total days since 1th of January 2000 | 0-65535 | days |  | Start Irrigation Date | Word | 0 |
| total days since 1th of January 2000 | 0-65535 | days |  | Stop Irrigation Date | Word | 2 |
| total days since 1th of January 2000 |  | days |  | Exceptions Days[0] | Word | 4 |
| total days since 1th of January 2000 |  | days |  | Exceptions Days[1] | Word | 6 |
| total days since 1th of January 2000 |  | days |  | Exceptions Days[2] | Word | 8 |
| total device time-window to irrigate union with irrigation programs limits | 0-65535 |  |  | Start Irrigation Hours | Word | 10 |
|  | 0-65535 |  |  | Stop Irrigation Hours | Word | 12 |
| timer interval to cancel all alerts, (0= don't cancel) | 0-65535 | minutes |  | Auto Reset | Word | 14 |
| time between closing | 0-65535 | seconds |  | Close Delay | Word | 16 |
| Master valve config [0,1,2,3]= [Deactivated, Auto, Paused, Always Open] |  |  |  | Define | Byte | 18 |
| 0 = parallel 1 = open main first, then valve 2 = open valve first, then main |  |  |  | Open Timing | Byte | 19 |
| 0 = parallel 1 = close main first, then valve 2 = close valve first, then main |  |  |  | Close Timing | Byte | 20 |
| time between opening | 0-255 | seconds |  | Open Delay | Byte | 21 |
| overlap/delay (bit Setup_B.Bit3) time between valves | 0-255 | seconds |  | Overlap Time | Byte | 22 |
| Values factor (in percent 0-200%) | 10-200 | % |  | Fix Water Multiply | Byte | 23 |
| 0= ignore. [1..CND_NUM] = condition number condition no. to point on (Cond Input) | 0-255 |  |  | Rain Condition Number | Byte | 24 |
| minimum days after last rain - before continue irrigation | 0-255 |  |  | Rain Delay Days | Byte | 25 |
| 0= ignore. [1..CND_NUM] = condition number condition no. to point on (Cond Input) |  |  |  | Pause Cond Number | Byte | 26 |
|  |  |  |  | Setup Byte A | Byte | 27 |
| 0 = parallel 1 = as queue (default) |  |  |  | Bit.0 – Programs Queue Operation |  | 27 |
| 0 = Fertilizer off 1 = Fertilizer on (default) |  |  |  | Bit.1 – Enable Fertilizer |  | 27 |
| 0 = not active 1 = active |  |  |  | Bit.2 – Is Local Sequence Active |  | 27 |
| 0= overlap, 1=delay (between valves) |  |  |  | Bit.3 – Parallel Time - Delay/Overlap |  | 27 |
|  |  |  |  | Bit.4 - |  | 27 |
|  |  |  |  | Bit.5 - |  | 27 |
|  |  |  |  | Bit.6 |  | 27 |
|  |  |  |  | Bit.7 |  | 27 |
|  |  |  |  | Setup Allow Message | Byte | 28 |
| 0 - disabled 1 – allow |  |  |  | Bit.0 – Allow Start Irrigation |  | 28 |
| 0 - disabled 1 – allow |  |  |  | Bit.1 – Allow Unfinished Irrigations |  | 28 |
| 0 - disabled 1 – allow |  |  |  | Bit.2 – Start Fert Messages |  | 28 |
| 0 - disabled 1 – allow |  |  |  | Bit.3 – Stop Fert Messages |  | 28 |
| 0 - disabled 1 – allow |  |  |  | Bit.4 – Start Flash Flow |  | 28 |
| 0 - disabled 1 – allow |  |  |  | Bit.5 – Stop Flash Flow |  | 28 |
| 0 - disabled 1 – allow |  |  |  | Bit.6 - |  | 28 |
| 0 - disabled 1 – allow |  |  |  | Bit.7 - |  | 28 |
|  |  |  |  | Setup Disable Days | Byte | 29 |
| N/A |  |  |  | Bit.0 |  | 29 |
| enable Sunday |  |  |  | Bit.1 |  | 29 |
| enable Monday |  |  |  | Bit.2 |  | 29 |
| enable Tuesday |  |  |  | Bit.3 |  | 29 |
| enable Wednesday |  |  |  | Bit.4 |  | 29 |
| enable Tuesday |  |  |  | Bit.5 |  | 29 |
| enable Friday |  |  |  | Bit.6 |  | 29 |
| enable Saturday |  |  |  | Bit.7 |  | 29 |
|  |  |  |  | Reserved | Word | 30 |

הגדרות תכנית השקיה

| CNF / Element Type:0x05 / Program |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Comments | Value Ranges | Units | Value type | Name | Len / Type | Offset |
| total days since 1th of January 2000 | 0-65535 | days |  | Start Irrigation Date | Word | 0 |
| total days since 1th of January 2000 | 0-65535 | days |  | Stop Irrigation Date | Word | 2 |
| min allowed time to irrigate 0 means - no limit | 0-65535 |  | hhh:mm | Start Irrigation Hours | Word | 4 |
| max allowed time to irrigate 0 means - no limit | 0-65535 |  | hhh:mm | Stop Irrigation Hours | Word | 6 |
| Cycled hours irrigation - start time | 0-65535 |  | hhh:mm | Start Time | Word | 8 |
| Cycled hours irrigation - cycle length | 0-65535 |  | hhh:mm | Cycle Time | Word | 10 |
| Cycled hours irrigation - stop time | 0-65535 |  | hhh:mm | Stop Time | Word | 12 |
| manual start hours | 0-65535 |  | hhh:mm | Hourly Starts[0] | Word | 14 |
| manual start hours | 0-65535 |  | hhh:mm | Hourly Starts[1] | Word | 16 |
| manual start hours | 0-65535 |  | hhh:mm | Hourly Starts[2] | Word | 18 |
| manual start hours | 0-65535 |  | hhh:mm | Hourly Starts[3] | Word | 20 |
| 2 bytes per valve (time or quantity) | 0-65535 |  | QUN0001 | Water Quant[0] | Word | 22 |
| 2 bytes per valve (time or quantity) | 0-65535 |  | QUN0002 | Water Quant[1] | Word | 24 |
| 2 bytes per valve (time or quantity) | 0-65535 |  | QUN0001 | Water Quant[2] | Word | 26 |
| 2 bytes per valve (time or quantity) | 0-65535 |  | QUN0001 | Water Quant[3] | Word | 28 |
| 2 bytes per valve (time or quantity) | 0-65535 |  | QUN0001 | Water Quant[4] | Word | 30 |
| 2 bytes per valve (time or quantity) | 0-65535 |  | QUN0002 | Water Quant[5] | Word | 32 |
| 2 bytes per valve (time or quantity) | 0-65535 |  | QUN0001 | Water Quant[6] | Word | 34 |
| 2 bytes per valve (time or quantity) | 0-65535 |  | QUN0001 | Water Quant[7] | Word | 36 |
| 2 bytes per valve (time or quantity) | 0-65535 |  | QUN0001 | Water Quant[8] | Word | 38 |
| 2 bytes per valve (time or quantity) | 0-65535 |  | QUN0002 | Water Quant[9] | Word | 40 |
| 2 bytes per valve (time or quantity) | 0-65535 |  | QUN0001 | Water Quant[10] | Word | 42 |
| 2 bytes per valve (time or quantity) | 0-65535 |  | QUN0001 | Water Quant[11] | Word | 44 |
|  | 0-65535 |  | QUN0001 | Fert Quant[0] | Word | 46 |
|  | 0-65535 |  | QUN0002 | Fert Quant[1] | Word | 48 |
|  | 0-65535 |  | QUN0001 | Fert Quant[2] | Word | 50 |
|  | 0-65535 |  | QUN0001 | Fert Quant[3] | Word | 52 |
|  | 0-65535 |  | QUN0001 | Fert Quant[4] | Word | 54 |
|  | 0-65535 |  | QUN0002 | Fert Quant[5] | Word | 56 |
|  | 0-65535 |  | QUN0001 | Fert Quant[6] | Word | 58 |
|  | 0-65535 |  | QUN0001 | Fert Quant[7] | Word | 60 |
|  | 0-65535 |  | QUN0001 | Fert Quant[8] | Word | 62 |
|  | 0-65535 |  | QUN0002 | Fert Quant[9] | Word | 64 |
|  | 0-65535 |  | QUN0001 | Fert Quant[10] | Word | 66 |
|  | 0-65535 |  | QUN0001 | Fert Quant[11] | Word | 68 |
|  | 0-65535 |  | QUN0001 | Water Before[0] | Word | 70 |
|  | 0-65535 |  | QUN0002 | Water Before[1] | Word | 72 |
|  | 0-65535 |  | QUN0001 | Water Before[2] | Word | 74 |
|  | 0-65535 |  | QUN0001 | Water Before[3] | Word | 76 |
|  | 0-65535 |  | QUN0001 | Water Before[4] | Word | 78 |
|  | 0-65535 |  | QUN0002 | Water Before[5] | Word | 80 |
|  | 0-65535 |  | QUN0001 | Water Before[6] | Word | 82 |
|  | 0-65535 |  | QUN0001 | Water Before[7] | Word | 84 |
|  | 0-65535 |  | QUN0001 | Water Before[8] | Word | 86 |
|  | 0-65535 |  | QUN0002 | Water Before[9] | Word | 88 |
|  | 0-65535 |  | QUN0001 | Water Before[10] | Word | 90 |
|  | 0-65535 |  | QUN0001 | Water Before[11] | Word | 92 |
|  | 0-65535 |  | QUN0001 | Water After[0] | Word | 94 |
|  | 0-65535 |  | QUN0002 | Water After[1] | Word | 96 |
|  | 0-65535 |  | QUN0001 | Water After[2] | Word | 98 |
|  | 0-65535 |  | QUN0001 | Water After[3] | Word | 100 |
|  | 0-65535 |  | QUN0001 | Water After[4] | Word | 102 |
|  | 0-65535 |  | QUN0002 | Water After[5] | Word | 104 |
|  | 0-65535 |  | QUN0001 | Water After[6] | Word | 106 |
|  | 0-65535 |  | QUN0001 | Water After[7] | Word | 108 |
|  | 0-65535 |  | QUN0001 | Water After[8] | Word | 110 |
|  | 0-65535 |  | QUN0002 | Water After[9] | Word | 112 |
|  | 0-65535 |  | QUN0001 | Water After[10] | Word | 114 |
|  | 0-65535 |  | QUN0001 | Water After[11] | Word | 116 |
| [0,1,2]=[ Deactivate, Active, Paused] |  |  |  | Define | Byte | 118 |
| 0 - lowest priority 1 - Regular. Won't stop other 2- Most Important. Will stop others if necessary |  |  |  | Priority | Byte | 119 |
| Array of participated valves [1...VLV_IN_PROGRAM] = [v1...vN] 0 - not available |  |  |  | Valve Number[0] | Byte | 120 |
| Array of participated valves [1...VLV_IN_PROGRAM] = [v1...vN] 0 - not available |  |  |  | Valve Number[1] | Byte | 121 |
| Array of participated valves [1...VLV_IN_PROGRAM] = [v1...vN] 0 - not available |  |  |  | Valve Number[2] | Byte | 122 |
| Array of participated valves [1...VLV_IN_PROGRAM] = [v1...vN] 0 - not available |  |  |  | Valve Number[3] | Byte | 123 |
| Array of participated valves [1...VLV_IN_PROGRAM] = [v1...vN] 0 - not available |  |  |  | Valve Number[4] | Byte | 124 |
| Array of participated valves [1...VLV_IN_PROGRAM] = [v1...vN] 0 - not available |  |  |  | Valve Number[5] | Byte | 125 |
| Array of participated valves [1...VLV_IN_PROGRAM] = [v1...vN] 0 - not available |  |  |  | Valve Number[6] | Byte | 126 |
| Array of participated valves [1...VLV_IN_PROGRAM] = [v1...vN] 0 - not available |  |  |  | Valve Number[7] | Byte | 127 |
| Array of participated valves [1...VLV_IN_PROGRAM] = [v1...vN] 0 - not available |  |  |  | Valve Number[8] | Byte | 128 |
| Array of participated valves [1...VLV_IN_PROGRAM] = [v1...vN] 0 - not available |  |  |  | Valve Number[9] | Byte | 129 |
| Array of participated valves [1...VLV_IN_PROGRAM] = [v1...vN] 0 - not available |  |  |  | Valve Number[10] | Byte | 130 |
| Array of participated valves [1...VLV_IN_PROGRAM] = [v1...vN] 0 - not available |  |  |  | Valve Number[11] | Byte | 131 |
| Array of participated valves [1...VLV_IN_PROGRAM] = [v1...vN] 0 - not available |  |  |  | More Valve[0] | Byte | 132 |
| Array of participated valves [1...VLV_IN_PROGRAM] = [v1...vN] 0 - not available |  |  |  | More Valve[1] | Byte | 133 |
| Array of participated valves [1...VLV_IN_PROGRAM] = [v1...vN] 0 - not available |  |  |  | More Valve[2] | Byte | 134 |
| Array of participated valves [1...VLV_IN_PROGRAM] = [v1...vN] 0 - not available |  |  |  | More Valve[3] | Byte | 135 |
| Array of participated valves [1...VLV_IN_PROGRAM] = [v1...vN] 0 - not available |  |  |  | More Valve[4] | Byte | 136 |
| Array of participated valves [1...VLV_IN_PROGRAM] = [v1...vN] 0 - not available |  |  |  | More Valve[5] | Byte | 137 |
| Array of participated valves [1...VLV_IN_PROGRAM] = [v1...vN] 0 - not available |  |  |  | More Valve[6] | Byte | 138 |
| Array of participated valves [1...VLV_IN_PROGRAM] = [v1...vN] 0 - not available |  |  |  | More Valve[7] | Byte | 139 |
| Array of participated valves [1...VLV_IN_PROGRAM] = [v1...vN] 0 - not available |  |  |  | More Valve[8] | Byte | 140 |
| Array of participated valves [1...VLV_IN_PROGRAM] = [v1...vN] 0 - not available |  |  |  | More Valve[9] | Byte | 141 |
| Array of participated valves [1...VLV_IN_PROGRAM] = [v1...vN] 0 - not available |  |  |  | More Valve[10] | Byte | 142 |
| Array of participated valves [1...VLV_IN_PROGRAM] = [v1...vN] 0 - not available |  |  |  | More Valve[11] | Byte | 143 |
| executes as time or quantity | 0-6 |  | QUN0001 | Water Units | Byte | 144 |
|  | 0-8 |  | QUN0002 | Fert Units | Byte | 145 |
| Cycled hours agitation - total cycles in day |  |  |  | Cycles Per Day | Byte | 146 |
| days interval for cyclic program |  |  |  | Cycle Days | Byte | 147 |
| Values factor (in percent 0-200%) | 10-200 |  |  | Fix Water Multiply | Byte | 148 |
| Bitmap (includes Weekly per) MSB - 1 weekly, 0 cyclic bits [0...6] = [Sun... Sat] |  |  |  | Irrigation Days | Byte | 149 |
| Bitmap |  |  |  | Setup Byte A | Byte | 150 |
| Is Hourly 0=interval 1=manual hours |  |  |  | Bit.0 Is Hourly interval |  |  |
| 0 - fertilizer off 1 - fertilizer on (default) |  |  |  | Bit.1 – Enable Fertilizer |  |  |
| N/A |  |  |  | Bit.2 |  |  |
| N/A |  |  |  | Bit.3 |  |  |
| N/A |  |  |  | Bit.4 |  |  |
| N/A |  |  |  | Bit.5 |  |  |
| N/A |  |  |  | Bit.6 |  |  |
| N/A |  |  |  | Bit.7 |  |  |
|  |  |  |  | Reserved | Byte | 151 |

הגדרות ברז

| CNF / Element Type:0x15 / Valve |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Comments | Value Ranges | Units | Value type | Name | Len / Type | Offset |
|  | 0-65535 | Based On Unit |  | Setup Flow | Word | 0 |
| 0 - deactivated 1 - active 2 - cancelled(ignore) |  |  |  | Define | Byte | 2 |
| (minutes) period to ignore flow alerts | 0-255 | minutes |  | Time Fill Delay | Byte | 3 |
| (%) lower flow deviation | 0-200 | % |  | Low Flow Dev | Byte | 4 |
| (%) upper flow deviation | 0-200 | % |  | High Flow Dev | Byte | 5 |
| (minutes) stability in low flow alert | 0-255 | minutes |  | Low Flow Fault Delay | Byte | 6 |
| (minutes) stability in high flow alert | 0-255 | minutes |  | High Flow Fault Delay | Byte | 7 |
| ASB |  |  |  | Low Flow React | Byte | 8 |
| ASB |  |  |  | High Flow React | Byte | 9 |
| ASB |  |  |  | No Pulse Fault React | Byte | 10 |
| ASB |  |  |  | No Pulse Fert React | Byte | 11 |
| 0 - not on Fert line 1 - on fertilizer line |  |  |  | Setup Byte A | Byte | 12 |
|  |  |  |  | bit 0 – Is Fertilizer Line |  |  |
|  |  |  |  | bit 1 |  |  |
|  |  |  |  | bit 2 |  |  |
|  |  |  |  | bit 3 |  |  |
|  |  |  |  | bit 4 |  |  |
|  |  |  |  | bit 5 |  |  |
|  |  |  |  | bit 6 |  |  |
|  |  |  |  | bit 7 |  |  |
|  |  |  |  | Reserved | Byte | 13 |

הגדרות שוטף מסננים

| CNF / Element Type:0x16/ Filter flush |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Comments | Value Ranges | Units | Value type | Name | Len / Type | Offset |
| water between washings | 0-65535 | m3 | 0-6553.5 | Water For Flash | Word | 0 |
| time between washings | 0-65535 |  | hhh:mm | Time For Flash | Word | 2 |
| 0 = deactivated 1 = activated |  |  |  | Define | Byte | 4 |
| 0 = ignore. [1..CND_NUM] = condition number condition no. to point on (Cond Input) |  |  |  | Start Cond | Byte | 5 |
| max allowed sequence washing requests |  |  |  | continuous | Byte | 6 |
| 0 = continue irrigation 1 = stop irrigation while washing |  |  |  | Stop Irrigation | Byte | 7 |
| ASB structure |  |  |  | Flash Failure React | Byte | 8 |
| time takes to fill pipe | 0-255 | minutes |  | Pipe Fill Delay | Byte | 9 |
| time length of washing | 0-255 | seconds |  | Unit Operation Time | Byte | 10 |
|  |  |  |  | Reserved | Byte | 11 |

הגדרות מדשנת

| CNF / Element Type:0x17 Fertilizer |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Comments | Value Ranges | Units | Value type | Name | Len / Type | Offset |
| pulse equalization | 0-65535 |  |  | Pulse Size | Word | 0 |
| according to Flow Type | 0-65535 | 0-65535 |  | Nominal Flow | Word | 2 |
|  |  |  | seconds | Fert Not Exe | Word | 4 |
| counting fertilizer failure time | 0-65535 |  | pulses | Leakage | Word | 6 |
| Indication for leakage, in pulses. | 0-65535 |  |  | Type | Byte | 8 |
| 0 - not active 1 - by counter 2 - by time |  |  |  | Fert Counter Input Number | Byte | 9 |
| fertilizer counter - input number | 0-4 |  |  | Pulse Type | Byte | 10 |
| 0=cc 1= liter 2=miligallon 3= gallon (In logs values will be as ###. #) | 0..N |  |  | Flow Type | Byte | 11 |
| 0=l/h (##. #) 1=gpm (##. #) 2=gph (##. #) |  | 25.5 | seconds | Pulse Time | Byte | 12 |
| length of pulse in time | 0-255 |  | seconds | Pipe Time Delay | Byte | 13 |
| delay before fertilizer starts |  |  |  | Setup Byte A | Byte | 14 |
|  |  |  |  | bit 0 – continuous Fert | Byte | 15 |
| 0 - not continuous 1 – continuous |  |  |  | bit 1 - |  |  |
|  |  |  |  | bit 2 |  |  |
|  |  |  |  | bit 3 |  |  |
|  |  |  |  | bit 4 |  |  |
|  |  |  |  | bit 5 |  |  |
|  |  |  |  | bit 6 |  |  |
|  |  |  |  | bit 7 |  |  |
| ASB structure |  |  |  | Leak Fert React | Byte | 16 |

הגדרות מד מים

| CNF / Element Type:0x18/ Water meter |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Comments | Value Ranges | Units | Value type | Name | Len / Type | Offset |
| For Fast flow - [k] |  |  |  | Data1 | Long | 6 |
| For Fast flow - [offset] |  |  |  | Data2 | Long | 10 |
|  | 0-65535 |  |  | Pulse Size | Word | 0 |
|  | 0-65535 |  | seconds | No Water Pulse Delay | Word | 2 |
|  | 0-65535 |  | pulses | Leakage Limit | Word | 4 |
|  | 0..4 |  |  | Input Number | Byte | 15 |
| 0-Regular 1-Fast | 0-1 |  |  | Type | Byte | 14 |
| 0=Liter 1= m3 2= Gallons (WOW) 3= TH Gallons (2*WOW) | 0..N |  |  | Pulse Type | Byte | 17 |
| 0=l/h 1=m3/h (##. #) 2=gpm 3=th-gph (##. #) | 0..N |  |  | Flow Type | Byte | 18 |
| ASB structure |  |  |  | Leak Water React | Byte | 16 |
|  |  |  |  | Reserved | Byte | 19 |

הגדרות לוגים יומנים:

| Logs / Log Type: 0x00 Irrigation Logs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Time Span | ID | Code | Element A | Series | Unit WQ | Unit WT | Unit FR | Unit FL | Reserved | Water time | Water QNT | Fert | Flow | CRC 16 | Total length |
| Offset | 0 | 4 | 6 | 7 | 8 | 9 | 9.5 | 10 | 10.5 | 11 | 12 | 14 | 16 | 18 | 20 |  |
| Length | 4 | 2 | 1 | 1 | 1 | 0.5 | 0.5 | 0.5 | 0.5 | 1 | 2 | 2 | 2 | 2 | 2 | 22 |
| סיום השקיה תקין |  |  | 0 | יחידת הספיקה | יחידת דשן | יחידת זמן | יחידת כמות | מס. סדרה | מס. מגוף |  | ספיקה מדודה | דשן בוצע | מים בוצע - כמות | מים בוצע - זמן |  |  |
| סיום השקיה לפי תנאי |  |  | 1 | יחידת הספיקה | יחידת דשן | יחידת זמן | יחידת כמות | מס. סדרה | מס. מגוף |  | ספיקה מדודה | דשן בוצע | מים בוצע - כמות | מים בוצע - זמן |  |  |
| סיום השקיה לפי שעות |  |  | 2 | יחידת הספיקה | יחידת דשן | יחידת זמן | יחידת כמות | מס. סדרה | מס. מגוף |  | ספיקה מדודה | דשן בוצע | מים בוצע - כמות | מים בוצע - זמן |  |  |
| ביטול השקיה |  |  | 3 | יחידת הספיקה | יחידת דשן | יחידת זמן | יחידת כמות | מס. סדרה | מס. מגוף |  | ספיקה מדודה | דשן בוצע | מים בוצע - כמות | מים בוצע - זמן |  |  |
| סיום השקיה ידני |  |  | 4 | יחידת הספיקה | יחידת דשן | יחידת זמן | יחידת כמות | מס. סדרה | מס. מגוף |  | ספיקה מדודה | דשן בוצע | מים בוצע - כמות | מים בוצע - זמן |  |  |
| סיום בגלל תנאי המתנה |  |  | 5 | יחידת הספיקה | יחידת דשן | יחידת זמן | יחידת כמות | מס. סדרה | מס. מגוף |  | ספיקה מדודה | דשן בוצע | מים בוצע - כמות | מים בוצע - זמן |  |  |
| סיום השקיה בתקלה |  |  | 6 | יחידת הספיקה | יחידת דשן | יחידת זמן | יחידת כמות | מס. סדרה | מס. מגוף | Alert Code | ספיקה מדודה | דשן בוצע | מים בוצע - כמות | מים בוצע - זמן |  |  |
| התחלת השקיה לפי שעות |  |  | 7 | יחידת הספיקה | יחידת דשן | יחידת זמן | יחידת כמות | מס. סדרה | מס. מגוף |  | ספיקה מתוכננת | דשן מתוכנן | מים נותר - כמות | מים נותר - זמן |  |  |
| התחלת השקיה לפי תנאי |  |  | 8 | יחידת הספיקה | יחידת דשן | יחידת זמן | יחידת כמות | מס. סדרה | מס. מגוף |  | ספיקה מתוכננת | דשן מתוכנן | מים נותר - כמות | מים נותר - זמן |  |  |
| התחלת השקיה ידני |  |  | 9 | יחידת הספיקה | יחידת דשן | יחידת זמן | יחידת כמות | מס. סדרה | מס. מגוף |  | ספיקה מתוכננת | דשן מתוכנן | מים נותר - כמות | מים נותר - זמן |  |  |
| המשך השקיה שלא סיימה |  |  | 10 | יחידת הספיקה | יחידת דשן | יחידת זמן | יחידת כמות | מס. סדרה | מס. מגוף |  | ספיקה מתוכננת | דשן מתוכנן | מים נותר - כמות | מים נותר - זמן |  |  |
| השהיית מגוף |  |  | 11 | יחידת הספיקה | יחידת דשן | יחידת זמן | יחידת כמות | מס. סדרה | מס. מגוף |  | ספיקה מדודה | דשן בוצע | מים בוצע - כמות | מים בוצע - זמן |  |  |
| התחלת דישון |  |  | 12 | יחידת הספיקה | יחידת זמן | יחידת זמן | יחידת כמות | מס. סדרה | מס. מגוף |  | ספיקה מדודה |  | מים בוצע - כמות | מים בוצע - זמן |  |  |
| סיום דישון |  |  | 13 | יחידת הספיקה | יחידת דשן | יחידת זמן | יחידת כמות | מס. סדרה | מס. מגוף |  | ספיקה מדודה | דשן בוצע | מים בוצע - כמות | מים בוצע - זמן |  |  |
| השהיית סדרה |  |  | 14 | יחידת הספיקה | יחידת דשן | יחידת זמן | יחידת כמות | מס. סדרה | מס. מגוף |  | ספיקה מדודה | דשן בוצע | מים בוצע - כמות | מים בוצע - זמן |  |  |

הגדרות לוגים התראות:

| Logs / Log Type: 0x01 Irrigation Logs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Time Span | ID | Code | Element A | Series | Unit WQ | Unit WT | Unit FR | Unit FL | Reserved | First data | Second data | Third data | Forth data | CRC 16 | Total length |
| Offset | 0 | 4 | 6 | 7 | 8 | 9 | 9.5 | 10 | 10.5 | 11 | 12 | 14 | 16 | 18 | 20 |  |
| Length | 4 | 2 | 1 | 1 | 1 | 0.5 | 0.5 | 0.5 | 0.5 | 1 | 2 | 2 | 2 | 2 | 2 | 22 |
| התחלת שטיפה מחושב לפי DP |  |  | 15 |  |  |  |  |  |  |  | מים עברו מאז שטיפה | זמן משטיפה קודמת |  |  |  |  |
| התחלת שטיפה לפי כמות |  |  | 16 |  |  |  |  |  |  |  | מים עברו מאז שטיפה | זמן משטיפה קודמת |  |  |  |  |
| התחלת שטיפה לפי שעות |  |  | 17 |  |  |  |  |  |  |  | מים עברו מאז שטיפה | זמן משטיפה קודמת |  |  |  |  |
| התחלת שטיפה ידנית |  |  | 18 |  |  |  |  |  |  |  | מים עברו מאז שטיפה | זמן משטיפה קודמת |  |  |  |  |
| סיום שטיפה רגיל |  |  | 19 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| סיום שטיפה ידני |  |  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| השהיית מערכת בתקלה |  |  | 21 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| השהיית מערכת לפי תנאי |  |  | 22 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| השהיית מערכת קבועה |  |  | 23 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| השהיית מערכת לדקות |  |  | 24 |  |  |  |  |  |  |  | דקות |  |  |  |  |  |
| ביטול תקלות ידני |  |  | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ביטול תקלות אוטומטי |  |  | 26 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| תיעוד מתח סוללה |  |  | 27 |  |  |  |  | Startup Volt(HB) | Startup Volt(HB) | Startup Volt(LB) | Vcap Volt | Modem Time Volt | Noon Volt | Midnight Volt |  |  |
| ניסיון תחילת תקשורת |  |  | 28 |  |  |  |  |  |  |  | סיבות - פירוט ב- Attempt Reason |  |  |  |  |  |
| סיום ניסיון תקשורת בצורה לא תקינה |  |  | 29 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| סיכום כמויות שגיאה כתיבה לכרטיסי זיכרון |  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| דיווח יומי לכמויות מים (מבוקר/לא מבוקר) |  |  | 31 |  |  |  |  |  |  |  | Uncontrolled water (old version - liter) | Total Daily water (old version - m3) |  |  |  |  |
| ריסט יחידה |  |  | 32 |  |  | uncontrolled water unit | total daily water unit |  |  |  |  |  |  |  |  |  |