City#Agent interface & protocol

Written By:	Assaf Bassi 9.3.2011

Updated By:	Eitan Revach

CONFIDENTIAL

The document is an introduction to Galcon Ltd. and is not an offer to sell, or a solicitation to buy an equity position Galcon. The information, data, drawings and the like contained in this document are proprietary to Galcon and strictly secret and confidential, and are supplied with the understanding that they will be held secret and confidential. The disclosure Galcon of information contained herein does not constitute any license or authorization to use or disclose information, ideas or concepts presented. No part of this document may be disclosed to any third party, copied, reproduced or stored on any type of media or  used in any way by any party without the express prior, written consent of Galcon its direct representatives. This document and all related materials must be returned to Galcon upon the Company’s request.

Contents

A. 	City#Agent Concept & Introduction	4

Concept:	4

Diagram:	4

Memory management:	5

Read from all types of memory	5

Real time (RT):	6

Configuration (CNF):	8

Logged data (LGD):	10

B. 	City#Agent Interface	12

C. 	Protocol Introduction	13

Packet Architecture:	13

Packet Shielding:	13

Bytes Order:	13

Workflow (starting flow for unit):	14

Possible routine Commands (in both directions)	14

D.	Commands - Server to Remote Unit	15

E.	Commands – Remote Unit To Server	24

F.	Formats	25

G.	Firmware burning	26

Updates:

| Comments | Version | Publisher | Date | No. |
| --- | --- | --- | --- | --- |
| Add function for sending events at “Real time” | 1.1 | Assaf. B | 12/8/2012 | 1 |
| Full Review | 2.0 | Eitan.R | 1 Dec 2015 | 2 |
|  |  |  |  | 3 |
|  |  |  |  | 4 |
|  |  |  |  | 5 |
|  |  |  |  | 6 |
|  |  |  |  | 7 |
|  |  |  |  | 8 |
|  |  |  |  | 9 |
|  |  |  |  | 10 |
|  |  |  |  | 11 |

# A. 	City#Agent Concept & Introduction

## Concept:

City#Agent is a” c type” library for embedded microcontroller that implements the entire communication interface between the device and the server. The main goal of the Agent is to give the customer full data access to the remote server without need to develop or create code for doing that.

## Diagram:

## Memory management:

The Agent shares 3 main types of memory with the application:

- Real time (RAM) – sector of sequence bytes in the volatile memory that used by the application for standard operation.

- Configuration (ROM/FLASH) – sectors of sequence bytes in Non-Volatile memory mainly for configuration data (application & Agent).

- Logged data (EEPROM, Flash etc) - sectors of sequence bytes in Non-Volatile memory mainly for logged data (application & Agent).

Read from all types of memory:

The application has freedom to organize all types of memory as it needs. The agent is not aware of how the memory is actually build and ordered by the application and therefore the agent need to have mechanism of how move inside the memory blocks.

The mechanism we use here is to send any request of read arriving from the server to the application. The application return the exact place where the agent shell start read.

The structure we use for this is:

Typedef enum {eMem_ROM, eMemRAM, eMemEE}eMemoryType;

Typedef struct

{

eMemoryType MemTyp;

unsigned char* Buftr;

int Len;

}stDataPtr;

## Real time (RT):

This part has been never implemented. Use Read/Write memory instead.

RT is a sector of N bytes (typically 100) that reflect the current status of the application. When the server needs to change the current status of the device, it will write into this sector. From the other hand, the device shell reflects any current status at this sector for keep the server updated.

The N bytes are divided to known structures of data that was pre-defined between the application and the server. Every request from the server or update coming from the device shell be refer in corresponded to those structured. The agent is not aware of the meaning of those structures.

Workflow process:

| Process | Source | Destination | Request | Comments |
| --- | --- | --- | --- | --- |
| RT Structure | Server | Agent | GetRT_Structures( Buffer, Len) | Return the buffer structures as predefined numbers. (e.g. [1,2,1,1,2,3,4,1]; When 1 is int, 2 is double etc.. ). The application is response for build this list. |
|  | Agent | Application | CA_ GetRT_Structures(stDataPtr* Dptr) | The application return pointer to buffer where the list is hold. |
|  | Agent | Server | Send_RT_Structure(Buf,Len) | The agent send the list back to the server |

| Process | Source | Destination | Request | Comments |
| --- | --- | --- | --- | --- |
| Read data | Server | Agent | ReadRT_Data(Offset, Len) | Read data from sector. Offset is calculated sequence from beginning of sector. |
|  | Agent | Server | Send_RT_Data(Buf,Len) | The agent send the data back to the server. No need to involve the application because it is only read process |

| Process | Source | Destination | Request | Comments |
| --- | --- | --- | --- | --- |
| Write data | Server | Agent | WriteRT_Data(Offset, Len ,DataBuffer) | Write data to sector. Offset is calculated sequence from beginning of sector |
|  | Agent | Application | Res = CA_ WriteRT_Data (Offset, Len ,DataBuffer) | The agent send request of writing to the application. The application can reject the request by sending back Res (result) > 0. Otherwise the response is OK. It is the application responsibility to reflect the write command in the RT sector |
|  | Agent | Server | Response_RT_Write(Res) | Send the application response to the write request. |

## Configuration (CNF):

CNF is a sector of N bytes that reflect the configuration data of the application and the agent. CNF typically written in ROM/FLASH memory for fast access by the application but in some cases can be written in the external EE.  When the server needs to change the current configuration of the device or the agent, it will write into this sector. From the other hand, the server can read the current configuration from the device.

The N bytes are divided to known structures of data that was pre-defined between the application and the server. Every request from the server or update coming from the device shell be refer in corresponded to those structures. The server assumes that every type of structure is hold sequentially in memory (e.g. if the application have 5 struct from type 9, it will hold it sequentially).

The server is not aware how the memory is physically organized and ordered by the application.

The agent is not aware of the meaning of those structures.

Workflow process:

| Process | Source | Destination | Request | Comments |
| --- | --- | --- | --- | --- |
| Read data | Server | Agent | ReadCNF_Data( type,Offset, Quant) | Ask for read quantity structures. Every struct from known type, start at offset (from first structure type appearance) |
|  | Agent | Application | CA_ GetCNF_Data (type,Offset, Quant ,stDataPtr* Dptr) | The application return pointer to buffer where the data is hold. |
|  | Agent | Server | Send_CNF_Data (type,Offset, Quant ,Buf) | The agent send the list back to the server |

| Process | Source | Destination | Request | Comments |
| --- | --- | --- | --- | --- |
| Write data | Server | Agent | WriteCNF_Data( type,Offset, Quant, Buf) | Ask for write quantity structures. Every struct from known type, start at offset (from first structure type appearance) with data buf. |
|  | Agent | Application | RES = CA_ WriteCNF_Data (type,Offset, Quant , Buf) | The application return success/reject for the writing operation. |
|  | Agent | Server | Write_CNF_Data (Res) | The agent send result to the server |

## Logged data (LGD):

LGD is a sector of N bytes that reflect the logged data and events of the application and the agent.  LGD is typically written in external EEPROM memory because the amount of data but in some cases can be written in the internal Flash. Only the Application and the agent are writing into LGD memory. The server can read the logged data in the device from time to time by asking.

The LGD memory can divide to several types of logs, every log has known unique type and his own data structure. Every request coming from the server shell be refer in corresponded to those structured and specific log type. The server assumed that every type of structure is hold sequentially in the memory as record ended with Date & Time.

The server is not aware how the memory is physically organized and ordered by the application. The agent is not aware of the meaning of those structures.

While the server send request to read log, the application can continue save records in the log. For avoiding collisions, the server will start by asking to freeze the log pointer for reading (writing of new record are not freeze). The server can do all the calculation of needed records according the “Freeze-Pointer”.

Workflow process:

| Process | Source | Destination | Request | Comments |
| --- | --- | --- | --- | --- |
| Prepare Read Log | Server | Agent | PrepareLog( LogType) | Ask for freeze the read pointer on the last record in specific log type. |
|  | Agent | Application | CA_ PrepareLog (LogType ,stDataPtr* LgStct) | The application return pointer to buffer where the log structure is hold. |
|  | Agent | Server | RespPrepareLog (LgStct) | The agent send the log struct back to the server |

| Process | Source | Destination | Request | Comments |
| --- | --- | --- | --- | --- |
| Read Log | Server | Agent | ReadLog(LogType,Offset, Quant) | Ask for read quantity of records from log of type LogType. Every record position is calculated from Freeze pointer (offset 0) backword. |
|  | Agent | Application | CA_ ReadLog (LogType, Offset, Quant , stDataPtr* LgStct) | The application return pointer to the place where the records are located. |
|  | Agent | Server | RespReadLog (LgStct) | The agent send result to the server |

# B. 	City#Agent Interface

# C. 	Protocol Introduction

## Packet Architecture:

| Closer | Header |  |  |  | Content |  | Closer |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ~ | TransactionID | ProtocolID | MasterType | Reserved | Function | Data | ~ |
| 1Byte | 2 Bytes | 1Byte | 1Byte | 2Byte | 1Byte | N Bytes | 1Byte |

Legend:

TransactionID	0x0000 – 0xFFFF – give by master for every message. Not checked by receiver.

ProtocolID	0x11. Slave should not answer for different protocol.

MasterType	Identify the first source for this message. 0x00=PC Master and 0x01=Unit Master.

Every answer should save the original MasterType sent by the sender.

Reserved	Always 0x00

## Packet Shielding:

0x7E should be only in begin and end of packets (as closer).

The values 0x7D and 0x7E must be shielded inside the packets using AND operator as follow:

0x7D		0x7D, 0x7D & 0xDF (=0x5D)

0x7E		0x7D, 0x7E & 0xDF (=0x5E)

## Bytes Order:

All protocol bytes should be as big-endian. (Except in particular commands when explicit mentioned).

## Workflow (starting flow for unit):

| Step No. | Caller (Msg Source) | Destination | Commands (see details bellow) |
| --- | --- | --- | --- |
| 1 | Unit | Server | 0x01 Connect |
|  | Server | Unit | 0x01 Connect Answer |
| 2 | Server | Unit | 0x05 Set Date/Time |
|  | Unit | Server | 0x05 Answer According to answer timing, server will may decide to disconnect unit. |
| 2 | Server | Unit | 0x06 Get Date/Time |
|  | Unit | Server | 0x06 Answer According to answer timing, server will may decide to disconnect unit. |
| 3 | Server | Unit | 0x42 Start send updates |
|  | Unit | Server | 0x42 Answer If failed, server disconnent unit. |

## Possible routine Commands (in both directions)

| 1 | Unit | Server | 0x11 Routine update from unit |
| --- | --- | --- | --- |
| 2 | Server | Unit | Any commands |

# D.	Commands - Server to Remote Unit

| Comments | Request / Response | Code | Function |
| --- | --- | --- | --- |
| Date/Time |  |  |  |
| Date/Time : All as Format D1 Answer: 0 = Success 1 = Failed | Request: Date Time structure; Day of Week; Reserved. 4Byte; 1byte; 3Byte. Reponse: Answer; Reserved. 1Byte; 2Byte. | 0x05 | Set Date/Time |
| Date/Time : All as Format D1 | Request: 00; 00. 1Byte; 1Byte. Reponse: Date Time structure; Reserved. 4Byte; 2Byte. | 0x06 | Get Date/Time |

| (CNF) Configuration Operations |  |  |  |
| --- | --- | --- | --- |
| Type of element: Describes separately in application docs. Element offset: The offset of element in sequence array. Element Quant: How many elements are send in the packets. Answer: 0x06 success | Request: Element Type; Element offset; Element quantity; Data Len; Data. 2bytes; 2 bytes; 1 byte; 2 bytes; N bytes. Reponse: Answer; Config ID; Reserved. 1Byte; 2 Bytes; 2 Bytes. | 0x20 | write CNF Data (No Paging!) |
| Answer: 0x06 success 0x01 No such element 0x02 – quant too big. | Request: Element Type; Element offset; Element quantity. 2bytes; 2 bytes; 1 byte. Reponse: Answer; Element Type; Element offset; Element quantity; Data Len; Data. 1 byte; 2bytes; 2 bytes; 1 byte; 2 bytes; N bytes. | 0x21 | read CNF Data (No Paging) |

| (LGD) Logger Data operations |  |  |  |
| --- | --- | --- | --- |
| Log type: Type of log Answer 06 – OK 03 – No log Log Structure: define the log structures that are in every record. | Request: Log type; 00. 2Bytes; 1Byte. Reponse: Answer; Structure sz; [Log Structure]; Reserved; Reserved. 1Byte; 2 Byte; N Bytes [Optional]; 1Byte; 1Byte. | 0x30 | Freeze logger pointer |
| Record size size in words, relevant for this packet only. Direction 1-forward 0-backward Offset - in records Status- 6 – ok 1 – invalid log struct 2 – no freeze point 3 – invalid request data 4-no record for send 5- the log is empty 7 – partial data, pls try again. | Request: Log type; Count; Direction; Offset HiHi; Offset Hi; Offset Low; Offset LowLow. 2Bytes; 1Byte; 1Byte; 1Byte; 1Byte; 1Byte; 1Byte. Reponse: status; Data; count; Last Memory Address; Reserved. 1byte; Count * Record size; 1Byte; 4Bytes; 2Byte. | 0x31 | Get n records from relatively to freezed pointer |

| Device Connection Operations |  |  |  |
| --- | --- | --- | --- |
|  | Request: 00; 00. 1Byte; 1 Byte. Reponse: - No Answer! | 0x50 | Disconnect |
|  | Request: 00; 00. 1Byte; 1 Byte. Reponse: - No Answer! | 0x4E | Reconnect |
|  | Request: 00; 00. 1Byte; 1 Byte. Reponse: - No Answer! | 0x4F | SOFT RESET |

| OTA – Remote Firmware Operations |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Request: Session ID – random number for current session. [1-0xFFFE] Response: 06 – Ready To Start 07 – Ready To Continue | Request: Session ID; File Size; Reserved. 2Bytes; 4Byte; 2Byte. Long (Bytes). Reponse: Status; Last Packet Index; Packet max size; Reserved. 1Byte; 2Byte; 2Byte; 2 Bytes. | 0x70 |  | Start download Firmware |  |
| Response: 06 – Ready 04 – Resend last Data len must be the same as in 0x70 | Request: Packet num; Data len; Data; Reserved. 2Byte; 2Byte; N Bytes; 2 Bytes. Reponse: Status; Reserved. 1Byte; 2 Bytes. | 0x71 |  | Get FW packet |  |
| Request: 06 – Burn FW Response: 06 – Ready to burn 06 – Resend all agai | Status. 1Byte. Request: Response: Request. 1Byte. | 0x72 |  | Finish download Firmware |  |
| (RT) Real-Time Operations |  |  |  |  |  |  |
| Address: XXXX – Real address at RAM. Data type: 1 – Byte (Use only low value). 2 – Word. Response: 06 – OK. |  | Request: Data len; Data; Reserved. Address 2 bytes; Value 2 bytes; Data type 1 byte; Reserved 1 byte. 1Byte; N Bytes; 2Byte. Reponse: Status; Reserved. 1Byte; 2 Bytes. | 0x78 |  | Write RT data |
| Request: Response: 06 – Read success |  | Request: Address; Data len; Reserved. 2 Byte; 2Byte; 2Byte. Reponse: Response; Address; Data len; Data; Reserved. 1 Byte; 2 Byte; 2Byte; N Bytes; 2Byte. | 0x79 |  | Read RT value |
| Address: XXXX – Real address at RAM. Element Type: 0 Bit 1 MSB as value Other bits as bit number >=1 Length as Element Response: 06 – OK. |  | Request: Element len (x); Number of elements; Data; Reserved. Address 2 bytes; Value x bytes. 1Byte; 1Byte; 2Byte. Reponse: Status; Reserved. 1Byte; 2 Bytes. | 0x80 |  | Write RT data V2 |

| Function outside protocol - Not Implemented yet |  |  |  |
| --- | --- | --- | --- |
| Look in table 1.1 << T.B.D >> | Request: Control; Params; Requ Len; Command. 1Byte; 4Byte; 1byte; N Bytes. Reponse: Status; Params; Answer Len; Answer. 1Byte; 4Byte; 1byte; N Bytes. | 0x48 | Transmit command to PLC |
| Type of element: Describes separately in application docs. Memory offset: Offset in entire data (all pages). Length(n): Current data lenght Answer: 0x06 success | Request: Constant over all paging packets; paging. Type; offset; Count; Memory offset; Length(n); Data. 2bytes; 2 bytes; 2 bytes; 2 bytes; 2 bytes; N bytes. Reponse: Answer; Reserved. 1Byte; 2 Bytes. | 0x25 | write CNF Data (Paging) |
| Answer: 0x06 success | Request: Constant over all paging packets; paging. Type; offset; Count; Memory offset; Length(n). 2bytes; 2 bytes; 2 bytes; 2 bytes; 2 bytes. Reponse: Answer; Len (N); Data. 1Byte; 2 Bytes; N Bytes. | 0x26 | read CNF Data (Paging) |
| Answer 06 ok 03-invalid db 04- invalid logs | Request: 00; 00. 1Byte; 1 Byte. Reponse: Answer; Reserved. 1Byte; 2 Bytes. | 0x4D | Start logging |

# E.	Commands – Remote Unit To Server

| Comments | Request / Response | Code | Function |
| --- | --- | --- | --- |
| Answer: 06 = Success | Request: Serial; Com Agent Version; App Version; Device model; Project Identifier; Max Packet Size; Reserved. 16 Bytes; 4Bytes; 4Bytes; 4Bytes; 2 Bytes; 2 Bytes; 8 Bytes. Reponse: Answer; Reserved. 1Byte; 2 Byte. | 0x10 | Connection request |
| Type: Type of event see table | Request: Type; Sub Type; Data Length; Data; Reserved. 1 Byte; 1 Byte; 1Byte; Length bytes; 2 Byte. Reponse: No Need! | 0x11 | Update send values |

# F.	Formats

Formats

Format D.1 – Date/Time format in 2+2 bytes

| Date : Year; month; Day; am/pm. 6bits; 4bits; 5bits; 1bit. Pm bit =1 when time equal/after 12:00 (in 24 hours clock) | Time : Seconds since 12:00. 2Bytes. |
| --- | --- |

Bytes Order for format D1:

| Date Hi | Date Low | Time Hi | Time Low |
| --- | --- | --- | --- |
| 1Byte | 1Byte | 1Byte | 1Byte |

# G.	Firmware burning

Concept

Burning firmware is done via the air by sending file of new firmware to the remote unit. The logical process is:

- Sending new firmware via the protocol (See above).

- Saving the new firmware at Non-Volatile memory (e.g. EEPROM).

- Raised flag of new firmware available.

- Reset the unit and start the boot loader.

- The boot loader recognize the flag and re-burn the new firmware into Micro.

- Firmware flag should be reset.

File format

The compiler compiled text file with firmware as ASCII format. The server shell read the text file and re-compiles it as binary file with the format bellow.

| Start address | Len | Data |
| --- | --- | --- |
| 2 bytes | 2 bytes | N bytes |

The final packet will be relevant address and Length of 0xFF. This type of packet will sign the EOF.